- An Najah National University , Electrical Eng. Dept.

- 2nd Semester 2009/2010  ,  15/4/2010   2nd   Exam

Course : Alternative Energy Systems

Instructor:  Prof. Dr. Marwan Mahmoud

Q1- A solar water heater ( SWH) with  a collector area of 1.33m2 has

7 riser pipes each of  1/2 inch-diameter, is designed to heat water

from 20 °C to 70 °C at a solar radiation intensity of 6.7kWh/m2-day.

- Sketch the efficiency curve of this SWH with appropriate values.

- Determine the volume of heated water (in litre ) during the mentioned solar day.

- What is the  water mass flow of this SWH (at the output 1 inch pipe)

- What is the appropriate size of hot storage tank of of this SWH ?

Q2- A  polycrystalline  PV module with 30 cells connected in series ,

each cell has an area of 50 cm2. If the NOCT of this module is 52°C,

the ambient temperature is 46°C and the solar radiation intensity on

the module surface is 920 W/m2,determine the followings:

- The IV-Curve of this module at the given conditions

- Fill Factor , Efficiency, Rs and Rsh of this module

Q3- A polycrystalline PV  module with 20 cells connected in series ,

each  cell of an area of 100Cm2 . A PV array consisting of this

module is used to charge 2 batteries connected in series where each

battery is rated at 12V/ 60 Ah.

- Sketch the IV Characteristics of this module at STC.

- If the daily solar energy intensity is 7.2 kWh/m2-day and the  Battery-DOD=65% , sketch the PV  array appropriate for charging fully the 2 batteries and the related charging time(in days).