- An Najah National University , Elect. Eng. Dept.

- 2nd  Semester 2009/2010  ,  1/3/2010   1st Exam

Course : Electric Machines

Instructor:  Prof. Dr. Marwan Mahmoud

-----------------------------------------------------------------------------------------

Q1-  The load  ZL = 16 < 30° Ω is supplied from a generator of VG =440< 0° V

(rms value) through a step up transformer ,a transmission line of

Zt = 0.8 +j 1.2 Ω , a step down transformer  as shown in Fig. 1 ,

determine the followings :

(a)-The voltage at the load VL , and the power consumed by the load PL

(b)-The voltage  drop on the line ΔV and the power loss in the line PT

(c)- The generator current IG and the power of the generator PG .

Q2- A  1000VA ,  230/115 V transformer has been tested to determine its

equivalent circuit. The tests were carried out on the primary side and had

the following  results :

Open Circuit –Test                                      Short Circuit-Test

Voc = 230 V                                                   Vsc = 10.8 V

Ioc  = 0.1 A                                                     Isc = 4.35  A

Poc = 5.2 W                                                    Psc =11.75 W

(a)   Find the equivalent circuit of the transformer referred to the secondary

side of the transformer and write on it the element values

- Determine the transformer voltage regulation at nominal conditions and

a lagging power factor of  PF = 0. 8

(c)  Determine the efficiency of the transformer