- An Najah National University , Elect. Eng. Dept.

- 2nd  Semester 2009/2010  ,  7/4/2010   2nd Exam

Course : Electric Machines

Instructor:  Prof. Dr. Marwan Mahmoud

-----------------------------------------------------------------------------------------

Q1- A four-pole DC- generator has an armature with  simplex  wave winding ,

has 51 slots ,with 12 conductors per slot is driven at 900 rpm . The useful

flux  per pole is 0.025 Vs.

(a)- Calculate the value of the generated voltage at no load .

(b)- If a 27 Ω resistor is connected to the terminals of this generator, what is the

induced torque on the shaft?

(c)- Considering (b) ,determine the power loss in each conductor of the armature.

Q2- A 10-pole 50 kW , 120 V  DC- generator has a duplex lap wound armature

which has 60 coils with 10 turns per coil. It's rated speed is 3600 rpm.

(a)- How much flux per pole is required to produce the rated voltage of this

generator at no load conditions ?

(b)- What is the current per path in the armature of this generator at rated load?

(c)-  What is the induced torque in this machine at rated load ?

(d)-  If the resistance of this winding is 0.01 Ω per turn, what is the armature

resistance RA of this machine?

Q3- A separately exited DC- generator is rated at 4kW,100V, 40A, IF =2.7A,

n=1800 rpm. It's magnetization curve (at n=1800rpm)is given as follows:

IF /A  =  1          2        2.2         2.3           2.4         2.5             2.7             4

EA /V  = 40       80        88          90           93          97              100           120

The machine has the following characteristics : RA = 0.25 Ω , RF=20 Ω

Radj = 0-100 Ω, VF = 80 V , NF = 800 turns per pol

(a)- If Radj =20Ω and the generator has 1400 rpm what is the no load voltage

VT ?

(b)- If the generator is loaded  by  RL = 15 Ω , determine the load current and

the power in the load .

(c) – If the generator has an armature reaction at full load of 250 A turns ,

determine  VT at IF = 2.7 A  , n =1600rpm  and   IL= 40 A