- An Najah National University , Elect. Eng. Dept.

- 2nd- Semester 2008/2009  ,  26/2/2009  1st  Exam

Course :Electrical Measurements &Instrumentation

Instructor:  Prof. Dr. Marwan Mahmoud

Q1-  The accuracy of a voltmeter is ± 2% at 10V full scale. If a potential

of  6V  is measured at a precision of 0.05V , determine the accuracy

of  this measurement.

Q2-  A wire rheostat consists of 1000 turns (Rtotal =100Ω ). If the sliding

contact touches only one turn by varying R from 0 to 100 Ω , what is

the possible measurable resolution ?

- Q3-  R1=100±4 Ω ,  R2=470±15Ω ,  R 3 = 560±28Ω . If  R1 is connected in

- series with the parallel connected  R2 and  R3 , determine the

- possible values of Rtotal with the relative error.

Q4-  R1 and R2 are connected in series and being supplied by a voltage

source of  V=24 VDC . If a voltmeter of a sensitivity S =  20kΩ/ V

and voltage range of 10 V is used to measure the voltage across R2

determine it's reading with the possible relative error.

(R1= 8kΩ ,   R2= 4kΩ ).

Q5- Use a PMMC of an internal resistance of  Rm = 100Ω and a max .

current of 75 mA , selector switch and resistaces to design an ayrton

shunt ammeter with the two ranges : I1=0… 200mA and

I2 =0…300mA . Sketch the ammeter cicuit and and calculate the

power ratings of the selected resistances .