- An Najah National University , Elect. Eng. Dept.

- 2nd- Semester 2008/2009  , 18/4/2009  Final Exam

Course :Electrical Measurements &Instrumentation

Instructors: Mr. Jamal Kharusheh ;  Prof. Dr. Marwan Mahmoud

Indicate the correct answer(s). There may be more than one correct answer. No marks for partially correct answers :

- The deflecting torque in a moving PMMC is proportional to :

a- flux               b-  I            c-  I2                        d-  Acoil

2- The deflecting torque in a moving  iron instrument is a function

of current as :

a- I-1                    b- I               c-  I2                        d- √I

3- A moving iron ammeter can be used to measure :

a- AC  current      b- Eddy current   c- DC current   d- Stray current

4- An electrodynamometer  instrument can measure :

a-AC current   b- DC current   c- Eddy current   d- AC&DC  power

5- The shunt resistor in a multi range ammeter has :

a- High value for higher current range

b- Low value  for higher current range

c- Same value as Rm of the PMMC

d- A value that allows Ishunt = Im

6- In a balanced three phase system , measuring power by two watt

meters  gives readings as W1 & W2 then the reactive power is :

a- W1+W2      b- W2-W1    c-(W1-W2 )2        d-√ (W22-W12 )

7- For  measuring coils of high quality factor we select :

a- Maxwell bridge   b- Sheering bridge  c- Hay bridge  d-Wien bridge

8- For measuring frequency and in harmonics analyzer we use  :

a- Maxwell bridge   b- Wien bridge  c- Hay bridge   d-Sheering bridge

9- The thermocouple instrument can be used as :

a- Watt meter  b- True rms meter  c- DC volt meter  d- DC ammeter

10- in a digital volt meter :

a- The voltage to be measured is converted  to frequency

b- The voltage to be measured is converted  to current

c- The voltage to be measured is converted  to time period

d- The voltage to be measured is converted  to pulse voltages

11-The phase shift between the two signals (Lissajous Figures ) is :

a- 0       b- 90    c- 135    d- 75

12-The frequency quotient fi/f2 for the Lissajous Figure is :

a- 1:2     b- 2:1   c- 3:2     d- 3:1

13- Relative humidity is measured by :

a- Thermometer  b- Pirani gage  c-Resistance hygrometer d-Capacitor

14- Liquid level and thickness are measured by :

a- Potentiometer b- Dielectric gage  c-AC coil  d- Piezoelectric pickup

15- Magnetic flux is measured by :

a- Reluctance pickup  b- Differential transformer  c- Eddy current gage

d- Hall effect sensor

16- Light and radiation is measured by  :

a- Photoconductive cell   b- Photovoltaic cell  c- Photo emissive cell

d- Thermometer

17-Velocity and vibration are measured by :

a- Moving coil generator  b- Hall effect generator  c-  Strain gage

d- Capacitor microphone

18- Force, torque and displacement are measured by :

a- Potentiometer               b- Strain gage                 c- Pirani gage

d- Differential transformer

19- A platinum resistance thermometer has 100 Ω at 25 C◦,

αT =0.00393Ω/C◦ , at  50 C◦  the resistance value is :

a- 102.7Ω          b- 112.3 Ω        c- 100.2 Ω                        d- 109.8 Ω

20- Temperature is measured by :

a- Photovoltaic cell  b- Photoconductive cell   c- Thermistor

d- Photo emissive cell