An-Najah National University

Chemical Engineering Department

Equipment Design

Design project # 3

Instructor: Eng. Shadi Sawalha                                                                   Due date: 12/5/2010

Distillation Column Design

120 kmoles/hour of a mixture of 5% propane, 15% i-Butane, 25% n-Butane, 20% i-Pentane and 35% n-Pentane is fed to a distillation tower at 8.3 bars. This mixture is to be fractionated so that 96% of the n-Butane is recovered in the distillate and 95% of i-Pentane is recovered in the bottom. The feed is at its boiling point

Design a distillation column to separate the mixture and calculate its capital, operating and total annual cost.