An Najah University – Faculty of engineering

Electrical Eng. Department

Instruments and Measurements

Instructor: Jamal Khrousheh                  17/4/2010                                        2nd  hr. exam

Question # 1 (5- marks)

A Maxwell bridge, designed to measure unknown impedance Ru , Lu of a coil is shown in figure below.

a- Derive an expression for Ru and Lu under balance conditions.

b- Calculate the Q factor for the coil if the supply frequency 50 Hz.

Question # 2 (10 marks)

What are the following bridges measured

a- Kelvin Bridge, Hay Bridge, Schering Bridge, Wien Bridge

b- What is the purpose of?

Intensity button, Focus button, Probe compensation, Attenuator

c- Draw block diagram for vertical stage for OSC. What is the main function of each stage?

Question # 3 (5 marks)

For the AC bridge shown, Knowing that R1= 100 ohm, C2= 100 uF  , Lx=  100 mH,  Rx= 200 ohm and wn = 1000 rad/sec, the bridge is unbalance, Where and what the is the value of a  variable resistance to be added to reach balance conditions