An Najah University – Faculty of engineering

Communication Eng. Department

System and Signals

Instructor: Jamal Khrousheh                                                          2nd. hr exam

15/4/2010

Question #1(10marks)

a) Compute the convolution for the following pairs of signals

x(t) = u(t+1) – u(t-3)

h(t)= e -2t u(t)

1

-1

1

X(jw)

b) Suppose that X (jw) is depicted in figure below

sketch the spectrum for y(t) given by:

1- y(t)= cos(t/2)x(t)

2- y(t)= cos(2t)x(t)

3- y(t)= d/dt(x(t))

Question # 2(10 marks)

a) The input and the output for a system related by the differential equation

d2y(t)dt2+6dy(t)dt+8y(t)=2x(t)

1- Find the impulse response of this system

2- What is the response of this system if x(t) = e-2t u(t)

b) Consider a system whose response to input

x(t) =  e-t u(t)

is

y(t) = [ 2e-t – 2 e-4t]u(t)

1-Determine the system impulse response

2- Find the differential equation relating the input and output of this system.